xm_hypo(l)        Misc. Reference Manual Pages         xm_hypo(l)

NAME

xm_hypo - Hypocenter format for the xmap8 program

SYNOPSIS

xm_hypo - Input File Format

DESCRIPTION

The default format for hypocenters in xmap8 is  UW  pickfile

format for hypocenters and focal mechanisms.

New: Now you can import hypoinverse  and  CalTech  Catalogue

hypocenters into xmap8.  See below.

Importation of fpfit output is still not available.

FORMAT

Can have Several Source Files input. Use: -S file1 -S file2.....

(S = Source)

The hypocenter format is determined by env variable: XMA_HYPO_FORMAT

which can be set to any of:  { UW , uw ,  hypoinverse , cit , CIT }

Or it can be changed on the fly on the command line by including

the suffix on the -S option, e.g.

-Suw files  -Scit file ...etc.

There is now a new format which is an augmented UW file

card that include the unique name of the file

added on the end, separated by a colon (:).

This allows you to access a database of digital waveforms

(or other information) which is accessed via the unique identification.

e.g.

setenv XMA_HYPO_FORMAT hypoinverse

the default is UW format, described below.

cit = CalTech Catalog format

hypoinverse = (one variant of) hypoinverse format

----------------------------------------

Hypocenters are normally screened spatially to limit

the number stored in memory.  Normally, hypocenters

located outside of the window surrounding the target

are excluded.  To force inclusion of all hypocenters

into memory use the command line option

-force_source

This will force all hypocenters to be included (no spatial selection).

SunOS 5.5           Last change: 5 March 1995                   1

xm_hypo(l)        Misc. Reference Manual Pages         xm_hypo(l)

This is important to use when applying the "connect" option on the Source menu.

This option requires that the source files being connected

have 1-1 correspondence.

---------UW FORMAT-----------------------

ident      sec   lat    long    depth mag np  ns gap delt err rms qq mod

A 9005051858 24.25 36N0127 117W5001  2.50  0.0 05/005 266 01 0.05 00.2AD F1

A 9005061311  7.80 36N0275 117W5144  2.02  0.2 06/005 304 05 0.03 00.3AD F1

A 9005061308 47.18 36N0005 117W4801  2.07  0.0 05/005 200 00 0.05 00.2AD F1

A 9005061309  5.68 36N0131 117W5024  2.08  0.0 06/006 273 02 0.03 00.1AD F1

See Details below.

Lat and Long are in the format ddNmmmm   where mmmm is minutes*100

N can be either N or S

W can be either W or E

#################################

FOCAL-MECHANISM file is also in UW format:

Can have Several focal Mechanism Files input. Use: -F file1 -F file2.....

It is organized as pairs of A and M cards.

The A cards are the same as those described above.

An example of an A & M-card pair is:

A 9204230225 29.57 33N5728 116W1868 12.23  3.3 82/000  63  7 0.18  0.2BA CN

M F 345 45 G 255 90 U 165 45 V 435  0 P 219 29 T 470 30

M:      strk1 dip1  strk2 dip2 pol1 dip1 pol2 dip2 p-axis pdip taxis t-dip

These pairs of cards must stay together and there may

be many concatenated in one file.

The first pair of strk-dip represent, by convention,

the focal plane and the second the auxiliary plane.

These can be switched by running a utility called switchfoc,

or they can be switched in xmap8.  See accelerator keys.

----------------------------------------------------------

Details of A-card -  UW FORMAT

A    The header line begins with a 'A' in the first col and

contains all of the date-time information for the

event, as well as location, magnitude and some quality

information.  The approved "types" which can appear in

column 2 are: X- Known explosion, P- Probable explo-

sion, F- Felt event, T- Teleseisms, H- Hand picked

(previous 'types' take precedence), L- Low frequency

volcanic, C- Centralia explosion, R- regional earthquake.

SunOS 5.5           Last change: 5 March 1995                   2

xm_hypo(l)        Misc. Reference Manual Pages         xm_hypo(l)

Quality factors (UW) :

Q1

C    =>     rms <0.5   err <5.0

B    =>     rms < 0.3  err < 2.5 errz<5

A    =>     rms <0.15  err< 1.0 errz < 2

Q2

C    =>     gap < 180  dmin < 50

B    =>     gap < 135   dmin < max(2*depth,5)

A    =>     gap <  90   dmin < max(depth,5)

Type,Year,Month,Day,Hour,Min,Sec,Lat,NS,Latmin*100,Longitude,EW,Lonmin*100

Depth,Fix,Magnitude,Numpobs,Numsobs,Gap,Mindelta,RMS,ERR,Q1,Q2,Velmodel

FORMAT('A',A1,5I2,F6.2,I3,A1,I4,I4,A1,I4,

F6.2,A1,F4.1,I3,'/',I3,I4,I3,F5.2,f5.1,2A1,1x,A2)

ATYYMMDDHHMM SS.SS LLNMMMM LLLWMMMM DD.DD* M.M NN/NNN GGG DD R.RR EE.EQQ VV

M    Mechanism line which summarizes the focal mechanism for

this event.  Up to 6 fix format packets in pairs for

nodal planes (F,G), slip vectors (U,V), and major

stress axes (P,T).

In format below AAA is the DOWN DIP AZIMUTH strike

of a plane measured clockwise from north eastward.

PP

is the dip measured from the horizontal downward.

For example a plane with a strike of N55E

and dipping 30 to the SSE is AAA=145 PP=30.

Plane#1,Plane#2,Slip#1,Slip#2,P-axis,T-axis,Comment

FORMAT('M',6(1x,A1,1x,I3,1x,I2),'C',A)

M F AAA PP G AAA PP U AAA PP V AAA PP P AAA PP T AAA PP C comment

-----------------------------------------------------------------------

hypoinverse example:

8005190858461646 1165122 1231  95219 17 62  2  29  0 0 190  0 0 19019JJJ  40  17  40  40 0

cit catalog example:

1977  1  1  20 22 52.44  34  3.32-116 31.53 C 0.0      5.63  0    0.00       0

SEE ALSO

xmap8 xm_well xm_wire xm_map xm_station xm_hypo xm_points

xm_origin xm_raster xm_color xm_menu

REFERENCES

Jonathan M. Lees

Yale University, Dept. Geology and Geophysics

SunOS 5.5           Last change: 5 March 1995                   3

xm_hypo(l)        Misc. Reference Manual Pages         xm_hypo(l)

P.O. Box 208109, New Haven, CT, 06520-8109

Voice: (203) 432-6947    FAX (203) 432-3134

email:  lees@lamb.geology.yale.edu

WWW:    http://love.geology.yale.edu/~lees

see article by J. M. Lees:

Lees, J. M. (1995). Xmap8: Three-dimensional GIS for geology and geophysics:

Seismological Research Letters: 66, 33-37.

SunOS 5.5           Last change: 5 March 1995                   4